EMBEDDED SYSTEMS

Multi-Mode ATmega32A Car Project

Students GROUP:

- Basel Mohamed Mostafa

- Ziad Refaie Abu AlFtooh

- Hussin Hesham Ali

- Andrew Emad Sadek

- Mohamed Ahmed Abdelnour

- Abdullah Sayed Mohamed

- Hassan Elkenany

- Fatma Ahmed Elawady

- Hagar Mohamed Abdelrahman

- Mariam Nashaat Boshra

- Mostafa Ayman

ABSTRACT

This report details the design and implementation of a custom robotic vehicle powered by an ATmega32A microcontroller. The system features a Finite State Machine (FSM) architecture that allows seamless, real-time transition between two distinct operating states: Manual Bluetooth Control and Autonomous Obstacle Avoidance. By developing a complete software stack from scratch, including the Microcontroller Abstraction Layer (MCAL) and Hardware Abstraction Layer (HAL) the project successfully demonstrates non-blocking peripheral management, hardware PWM signal generation, and environmental spatial awareness.

Contents

Table of Figures	3

Chapter 1: INTRODUCTION	4

1.1	Overview:	4

1.2 Scope of the Project	4

Chapter 2: Hardware	5

2.1 The Microcontroller	5

2.2 Actuators & Motor Control	5

2.3 Sensors & Communication	5

2.4 Power Management	5

Chapter 3: Software & Architecture	7

3.1 Layered Architecture Overview	7

3.2 Peripheral Utilization	7

3.3. MOTOR Control Functions	8

In Conclusion:	10

Chapter 4: System Operating Modes	11

4.1 Mode 0: Manual Bluetooth Control	11

4.1.1 Bluetooth Control Function:	11

4.2 Mode 1: Autonomous Obstacle Avoidance	12

4.2.1 Servo Functions:	12

4.2.2 Servo Set Angle function structure	13

4.2.3 Autonomous control function	14

4.3. LCD Control Functions	17

4.3.1 LCD display string function	17

4.3.2 LCD display character function	18

Chapter 6: Complete Driver ReadMEs	21

6.1 DIO Driver	21

Chapter 6: Conclusion	50

6.1 Conclusion	50

# Table of Figures

Figure 1: Building the Chassis Car.	4

Figure 2: Finished Car structure.	5

Figure 3: 18650 Batteries.	6

Figure 4: Showcasing Library structure	7

Figure 5: All motor control functions.	8

Figure 6: Motor_Forward Function internal structure.	8

Figure 7: DIO_setPinValue function internal code.	9

Figure 8: L298D structure.	10

Figure 9: Bluetooth Control Function	11

Figure 10: Autonomous Routines & Sensors for the second control mode.	12

Figure 11: Servo Functions Structure.	13

Figure 12: Servo main angle moving and control function with Timer 2.	13

Figure 13: Timer Set Compare function explanation figure for Timer 2 peripheral.	14

Figure 14: Main Autonomous Control function for the second mode.	16

Figure 15: Inner structure of the Robot_UpdateLCD function.	17

Figure 16: Structure of LCD display string function.	17

Figure 17: LCD_displayChar function to display characters on the LCD 16x2.	18

Figure 18: Function to send proper commands to the LCD.	19

Figure 19: Writing Data using only 4 bit mode for the 16x2 LCD display.	19

# Chapter 1: INTRODUCTION

## Overview:

Robotic vehicles are frequently deployed in environments that are either inaccessible or hazardous to humans. Depending on the scenario, these robots must either be teleoperated by a human driver or possess the intelligence to navigate independently. This project bridges both paradigms by creating a unified robotic platform capable of remote exploration and autonomous environmental reaction.

The primary objective of this project was to engineer a reliable embedded system capable of:

- Processing non-blocking wireless commands via Bluetooth.

- Generating precise pulse-width modulation (PWM) signals for DC motor speed control and servo positioning.

- Utilizing ultrasonic time-of-flight data to map immediate surroundings and avoid collisions.

- Providing real-time system feedback via an LCD interface.

## 1.2 Scope of the Project

Figure 1: Building the Chassis Car.

The project encompasses both hardware and software development. Hardware tasks included chassis assembly, power distribution, and component wiring. Software tasks involved writing pure C drivers for the ATmega32A registers, establishing layered software architecture, and writing the application-level FSM logic to safely manage state transitions.

# Chapter 2: Hardware

## 2.1 The Microcontroller

The brain of the system is the ATmega32A 8-bit AVR microcontroller, clocked externally at 16MHz. This MCU was selected for its robust suite of hardware timers, standard communication protocols (USART), and sufficient GPIO pins to handle multiple actuators and sensors simultaneously.

## 2.2 Actuators & Motor Control

- DC Motors & L298N Driver: The vehicle utilizes a four-wheel skid-steering drive system. Power is routed through an L298N Dual H-Bridge motor driver, allowing the MCU to control both the direction and speed of the left and right motor banks.

- SG-90 Micro Servo: Mounted at the front of the vehicle, this servo acts as a panning "neck" for the ultrasonic sensor, allowing the robot to scan its environment left and right.

Figure 2: Finished Car structure.

## 2.3 Sensors & Communication

- HC-05 Bluetooth Module: Connected to the MCU’s hardware UART pins, this module receives serial commands from an Android controller application at a baud rate of 9600.

- HC-SR04 Ultrasonic Sensor: This acoustic sensor acts as the robot's "eyes," calculating the distance to nearby obstacles by measuring the time-of-flight of high-frequency sound pulses.

## 2.4 Power Management

The system is powered by four 18650 lithium-ion batteries arranged as two separate two-cell power sources. This configuration provides stable power distribution while helping isolate the sensitive ATmega32A control

Figure 3: 18650 Batteries.

circuitry from the high-current demands of the DC motors and the SG-90 servo. The L298N module’s onboard 5V regulator is used to supply logic-level components.

- Motor power supply: Two 18650 batteries power the DC motors and the L298N motor driver module.

- Control power supply: Two 18650 batteries power the ATmega32A main board and the LCD.

# Chapter 3: Software & Architecture

## 3.1 Layered Architecture Overview

To ensure code reusability and maintainability, the software was strictly divided into layers:

- MCAL (Microcontroller Abstraction Layer): Custom drivers for DIO, TIMER, and USART peripherals.

- HAL (Hardware Abstraction Layer): Component-specific drivers for the LCD, L298N Motors, HC-SR04, and SG-90.

Figure 4: Showcasing Library structure

Application Layer: The FSM and primary logic loops executing in main.c.

In the above figure, we see he utilization of two important MCAL and two important HAL libraries being used.

MCAL:

- The DIO drivers used.

- The USART driver for wireless communication and control via Bluetooth module HC-05.

HAL:

- The HC-SR04 ultrasonic driver for distance measurement.

- The Servo SG-90 driver for controlling the direction that the HC-SR04 faces and measures the distance in.

## 3.2 Peripheral Utilization

Efficient resource management was critical for multi-mode operation:

- Timer 0: Configured in Fast PWM mode with a prescaler of 64 to generate the duty cycles required for dynamic DC motor speed control.

- Timer 2: Configured as an 8-bit Fast PWM generator to output a ~61Hz control signal for the SG-90 servo, freeing up the 16-bit Timer 1 for other high-precision tasks.

- USART (Interrupt-Driven): Configured to receive data asynchronously (USART_receiveByteNonBlocking). This ensures the CPU is never stalled waiting for user input, which is critical for the FSM's stability.

- DIO: This is the main Input/Output driver for controlling the direction of the pins and ports present in the ATmega32A.

## 3.3. MOTOR Control Functions

Figure 5: All motor control functions.

The motor control module provides five main functions:

- A function to turn the vehicle left.

- A function to turn the vehicle right.

- A function to drive all motors forward.

- A function to stop all motors and halt the vehicle.

- A function to reverse the motor direction and move the vehicle backward.

Motors_Forward(u8 copy_u8Speed) is one of the key functions in this module.

Figure 6: Motor_Forward Function internal structure.

This function uses the DIO peripheral to control the ATmega32A input/output pins directly. By setting specific pins to HIGH (5V) or LOW (GND), it determines the direction of motor rotation.

One of the most important functions used here is DIO_setPinValue.

The DIO_setPinValue function is a core part of the Microcontroller Abstraction Layer (MCAL). Its role is to write a digital state, either HIGH or LOW, to a specific ATmega32A pin without affecting the other pins on the same port.

Parameters

- copy_u8Port: The target port, such as PORTA or PORTB.

- copy_u8Pin: The specific pin number, from 0 to 7.

- copy_u8Value: The required digital state, either DIO_PIN_HIGH or DIO_PIN_LOW.

Detailed Execution Flow & Hardware Protection

Before accessing any hardware register, the function validates its input parameters. It calls validatePinParams() to confirm that the selected port and pin exist on the microcontroller, and it checks that the requested value is either logical HIGH or LOW. If any parameter is invalid, the function immediately stops and returns an error state, helping prevent hardware faults or memory corruption.

Figure 7: DIO_setPinValue function internal code.

Instead of relying on a long switch-case statement to locate the correct hardware register, the function uses a pointer lookup table: portReg = portRegisters[copy_u8Port];. This approach retrieves the address of the required PORT register efficiently and improves execution speed.

### In Conclusion:

All other functions follow similar code workings, in the fact that they are controlled by the DIO peripheral and the pins are manipulated according to the L298D structure.

Figure 8: L298D structure.

All Four motors are connected according to the instructions present in the above figure.

# Chapter 4: System Operating Modes

## 4.1 Mode 0: Manual Bluetooth Control

In this state, the robot acts as a standard teleoperated vehicle. The USART peripheral parses ASCII characters ('F', 'B', 'L', 'R', 'S') sent from the smartphone application. The application layer routes these commands to the Motor HAL to execute forward, reverse, and pivot-turn maneuvers. The 16x2 LCD is continuously updated to display the current connection status and active movement command.

### 4.1.1 Bluetooth Control Function:

Figure 9: Bluetooth Control Function

This function is basically the main control method for switching modes in the vehicle AND for controlling via the Arduino Bluetooth Controller APP on mobile.

- It parses the incoming characters via Mobile APP and currently has two characters for MODE change and FIVE characters for Bluetooth RC control.

The function structure is as follows:

The function waits for characters via a USART loop as shown below.

- This function waits for incoming characters in a USART loop and verifies that each character is received correctly before continuing. If the function returns E_OK, the character was received successfully and the program proceeds. If it returns E_NOT_OK or E_TIMEOUT, an error has occurred, and the function is not executed.

The Bluetooth control function interprets each received character and maps it to a specific motor action, as follows:

- F: Calls Motors_forward(speed) to drive all motors forward.

- B: Calls Motors_reverse(speed) to drive all motors in reverse.

- L: Calls Motors_Left(speed) to turn the vehicle left.

- R: Calls Motors_right(speed) to turn the vehicle right.

- S: Stops all motors and places the vehicle in standby mode.

## 4.2 Mode 1: Autonomous Obstacle Avoidance

Upon receiving the override command ('2'), the FSM shifts into autonomous operation.

- Cruising: The robot drives forward continuously while pinging the HC-SR04.

- Detection & Braking: If an object is detected within 25cm, the robot immediately halts, reverses for 400ms to clear physical space, and stops.

- Scanning: The SG-90 servo sweeps the sensor to 0° (Right) and 180° (Left), taking distance readings at each vector.

- Decision Making: The MCU compares the left and right distance variables and steers the motors toward the most open path. If both paths are blocked, a 180° U-turn failsafe is executed.

### 4.2.1 Servo Functions:

These functions are the main operations that the servo SG-90 performs:

- Robot_LookRight, which controls the servo to move the head of the vehicle to look right.

- Robot_LookLeft, which controls the servo to move the head of the vehicle to look left.

Figure 10: Autonomous Routines & Sensors for the second control mode.

Autonomous_Routine, The main function for the autonomous routine (servo + logic).

As examples, consider the Robot_LookRight and Robot_LookLeft functions. Both routines use the SG-90 servo driver to position the sensor by calling SERVO_setAngle_Timer2(ANGLE), then measure the surrounding distance using HCSR04_getDistance(&dist).

Each function includes a 500-millisecond delay to give the servo enough time to reach the target position. Once the movement is complete, the system reads the distance from the ultrasonic sensor to obtain an accurate measurement in the selected direction.

Figure 11: Servo Functions Structure.

### 4.2.2 Servo Set Angle function structure

The interaction between SERVO_setAngle_Timer2 and TIMER_setCompare perfectly demonstrates the project's layered software architecture. The system separates physical logic (manipulating real-world degrees) from silicon logic (manipulating internal memory registers). The HAL handles the physics and math, while the MCAL handles the hardware execution.

Step 1: The HAL Layer (SERVO_setAngle_Timer2) This function sits in the Hardware Abstraction Layer. Its job is to translate human-readable intentions into machine-readable data.

- Safety Clamping: It first checks if (copy_u8Angle > 180). This is a physical hardware protection mechanism to ensure the user (or the FSM) cannot accidentally command the SG-90 servo to rotate past its mechanical limits, which would strip its internal plastic gears.

- Linear Mapping: The ATmega32A's Timer 2 does not understand "degrees." It only understands clock ticks. This function uses a linear mapping algorithm to convert the 0°–180° range into the strictly defined 9–38 tick range required to generate the ~0.6ms to ~2.4ms PWM pulse widths.

- The Handoff: Once the exact hardware tick value (compareValue) is calculated, it passes the raw number down to the MCAL via TIMER_setCompare().

Figure 12: Servo main angle moving and control function with Timer 2.

Step 2: The MCAL Layer (TIMER_setCompare) This function sits in the Microcontroller Abstraction Layer. It knows absolutely nothing about servos, angles, or robots. Its only job is to safely write a value into a timer's Output Compare Register.

Figure 13: Timer Set Compare function explanation figure for Timer 2 peripheral.

Validation: It first verifies that TIMER2 and TIMER_CHANNEL_A are valid inputs, returning an error state if a higher layer passes a faulty parameter.

- Atomic Protection: Just like the digital I/O drivers, writing to timer registers can be vulnerable to race conditions. The function saves the Global Status Register (SREG) and disables interrupts (cli()) to ensure the timer register is updated safely without interruption.

- Register Execution: Finally, a switch-case routes the value to the exact silicon memory address. For Timer 2, it casts the 16-bit value down to 8-bit and assigns it to OCR2_REG (Output Compare Register 2).

### 4.2.3 Autonomous control function

The Autonomous Routine function serves as the central "brain" of the robot when operating in MODE_AUTONOMOUS. It resides purely in the Application Layer, abstracting away all hardware registers. Instead, it relies entirely on the Hardware Abstraction Layer (HAL) to poll sensors, update the LCD dashboard, sweep the servo, and command the motor driver, effectively creating a reactive Finite State Machine (FSM).

1. Environmental Polling & Cruising State The function begins by polling the HC-SR04 ultrasonic sensor. If the sensor returns a valid reading (HCSR04_OK) and the path ahead is clear (distance > 25cm), the system enters its standard cruising state. It ensures the servo "head" is facing straight ahead (90°) and commands the DC motors to drive forward continuously.

2. Kinematic Dampening (Braking & Reversing) If an obstacle is detected within the 25cm threshold, the robot immediately initiates its evasion sequence. Because physical robots have momentum, the function does not attempt to turn immediately. Instead, it applies a "Kinematic Dampening" sequence:

- It cuts power to all motors (Motors_off()) and waits 200ms to bleed off forward momentum.

- It then commands the motors in reverse for 400ms. This is a critical mechanical failsafe that prevents the chassis's front corners from scraping against the obstacle during the subsequent pivot turn.

3. Spatial Sweeping (Environmental Mapping) Once backed away from the obstacle, the robot maps its immediate surroundings to find an escape route. It calls the helper functions Robot_LookRight() and Robot_LookLeft(), which command the SG-90 servo to sweep the ultrasonic sensor to 0° and 180°. Crucially, these helper functions contain necessary mechanical delays to allow the plastic gears to reach their destination before the acoustic ping is fired.

4. Decision Making With the spatial data collected, the MCU executes a heuristic comparison:

- It compares distanceRight against distanceLeft and steers the robot toward the most open path, provided that path has at least 20cm of clearance.

- The Dead-End Failsafe: If both the left and right paths return distances less than 20cm (indicating the robot has driven into a tight corner or box), the FSM triggers a U-Turn override, spinning the robot to the right for double the standard turning duration (700ms) to face the opposite direction and escape.

- Acoustic Overlap Prevention At the very end of the routine, an explicit _delay_ms(50) is executed. Ultrasonic sensors operate by listening for the acoustic echo of their own ping. In an enclosed room, sound waves bounce multiple times. This 50ms delay ensures that all residual sound waves from the previous measurement have dissipated, preventing false-positive readings caused by "echo overlap."

Figure 14: Main Autonomous Control function for the second mode.

## 4.3. LCD Control Functions

Figure 15: Inner structure of the Robot_UpdateLCD function.

The main LCD function used to update the LCD 16x2 display of the current mode selected and the command received.

The LCD control routine relies on two core functions from the LCD driver to manage what appears on the 16×2 display and where it is shown:

LCD_displayString: Displays text on the LCD screen.

LCD_setCursor: Sets the starting row and column for the next text displayed on the LCD.

Figure 16: Structure of LCD display string function.

4.3.1 LCD display string function

The LCD_displayString function is responsible for writing an entire text string to the LCD in a controlled and reliable way.

Main steps:

- It first checks the input string for invalid or unsupported characters, such as escape characters or other symbols that should not be sent to the display.

- If the string is valid, the function processes it one character at a time.

- For each character, it calls LCD_displayChar in a loop until the full string has been shown on the LCD.

### 4.3.2 LCD display character function

The LCD driver utilizes a strict top-down call hierarchy to transmit characters to the screen. To the user (the Application Layer), printing a character requires a single, simple function call. Internally, the Hardware Abstraction Layer (HAL) cascades this request through three distinct functions, peeling back a layer of hardware complexity at each step: LCD_displayChar ➔ LCD_writeByte ➔ LCD_writeData4Bit.

Figure 17: LCD_displayChar function to display characters on the LCD 16x2.

Step 1: The Public API (LCD_displayChar) This is the only function exposed to the main application. Its job is application-level state management.

- It accepts an ASCII character (e.g., 'A') and passes it to the lower-level byte writer, explicitly setting the Register Select (RS) flag to 1 to indicate that this byte is visual data, not a system command.

- Cursor Tracking: Crucially, after sending the byte, this function mathematically updates the internal LCD_currentCol and LCD_currentRow variables. This software tracking allows the driver to automatically wrap text to the next line if the cursor reaches the edge of the physical screen.

Step 2: The Preprocessor Router (LCD_writeByte) This static (private) function acts as the internal traffic controller. It knows that LCDs can be physically wired in either 8-bit mode (using 8 data pins) or 4-bit mode (using 4 data pins to save microcontroller resources).

Figure 18: Function to send proper commands to the LCD.

Compile-Time Configuration: Instead of using runtime if/else statements that waste CPU cycles, it uses C preprocessor directives (#if LCD_MODE == LCD_MODE_4BIT).

- When you compile the code, the compiler looks at your configuration and permanently routes the byte to the correct hardware execution function (LCD_writeData4Bit), discarding the unused 8-bit code to save flash memory space. It also handles universal delays required by the HD44780 LCD controller chip.

Step 3: The Hardware Execution (LCD_writeData4Bit) This function represents the lowest level of the HAL, physically toggling the ATmega32A's silicon pins to communicate with the LCD screen.

Figure 19: Writing Data using only 4 bit mode for the 16x2 LCD display.

Pin Configuration: It first sets the RS pin (Command vs. Data) and pulls the Read/Write (RW) pin LOW to put the LCD into Write mode.

- The 4-Bit Multiplexing: Because the system is operating in 4-bit mode to save GPIO pins, an 8-bit character cannot be sent all at once. The function splits the byte in half:

  - It takes the top 4 bits (the "high nibble") using a bitwise right-shift (copy_u8Data >> 4), applies them to the data pins, and fires the LCD_enablePulse() to latch the data into the screen.

  - It then takes the bottom 4 bits (the "low nibble"), applies them to the pins, and fires a second enable pulse.

# Chapter 6: Complete Driver ReadMEs

## 6.1 DIO Driver

ATmega32A DIO Driver Documentation

Table of Contents

- Overview

- Features

- Hardware Architecture

- Integration Guide

- API Reference

- Code Examples

- Error Handling

- Best Practices

- Troubleshooting

- Migration Guide

Overview

This Digital Input/Output (DIO) driver provides a comprehensive interface for controlling the GPIO pins on the ATmega32A microcontroller. It serves as the foundation for all other peripheral drivers (USART, LCD, etc.) and offers both pin-level and port-level operations with professional-grade error handling and atomic operations.

Why This Driver?

- Robust: Full parameter validation and error checking

- Safe: Atomic operations prevent corruption during interrupts

- Efficient: Register lookup tables instead of slow switch-cases

- Flexible: Both pin and port-level operations

- Professional: Comprehensive error handling and return types

Features

- ✅ Individual pin control (direction, value, toggle, read)

- ✅ Full port operations (direction, value, toggle, read)

- ✅ Internal pull-up resistor configuration

- ✅ Masked port operations (modify specific bits)

- ✅ Configuration structures for批量 initialization

- ✅ Atomic operations with interrupt protection

- ✅ Comprehensive error handling

- ✅ Input parameter validation

- ✅ Register lookup tables for efficiency

- ✅ Utility functions for validation

Hardware Architecture

ATmega32A Ports

| Port | Pins | Registers | Purpose |
| --- | --- | --- | --- |
| PORTA | PA0-PA7 | DDRA, PORTA, PINA | General purpose |
| PORTB | PB0-PB7 | DDRB, PORTB, PINB | General purpose + special functions |
| PORTC | PC0-PC7 | DDRC, PORTC, PINC | General purpose |
| PORTD | PD0-PD7 | DDRD, PORTD, PIND | General purpose + special functions |

Register Overview

- DDRx: Data Direction Register (0=input, 1=output)

- PORTx: Output Data Register (sets output value or pull-up)

- PINx: Input Pin Address (reads pin values)

Integration Guide

1. File Structure

your_project/

│

├── UTILS/

│   ├── std_types.h

│   └── bit_utils.h

│

├── MCAL/

│   ├── DIO_REG.h

│   ├── DIO_PROGRAM.h

│   └── DIO_PROGRAM.c

│

└── main.c

2. Dependencies

Ensure these utility files are present:

- std_types.h - Standard type definitions (u8, bool_t, etc.)

- bit_utils.h - Bit manipulation macros (SET_BIT, CLR_BIT, etc.)

3. Basic Setup

#include "MCAL/DIO_PROGRAM.h"

int main(void) {

// Your code here

return 0;

}

API Reference

Constants and Types

Port Definitions

#define DIO_PORTA           0

#define DIO_PORTB           1

#define DIO_PORTC           2

#define DIO_PORTD           3

Pin Definitions

#define DIO_PIN0            0

#define DIO_PIN1            1

#define DIO_PIN2            2

#define DIO_PIN3            3

#define DIO_PIN4            4

#define DIO_PIN5            5

#define DIO_PIN6            6

#define DIO_PIN7            7

Direction and Value Constants

// Pin Direction

#define DIO_PIN_INPUT       0

#define DIO_PIN_OUTPUT      1

// Pin Value

#define DIO_PIN_LOW         0

#define DIO_PIN_HIGH        1

// Port Direction

#define DIO_PORT_INPUT      0x00

#define DIO_PORT_OUTPUT     0xFF

// Port Value

#define DIO_PORT_LOW        0x00

#define DIO_PORT_HIGH       0xFF

// Pull-up Configuration

#define DIO_PULLUP_DISABLE  0

#define DIO_PULLUP_ENABLE   1

Error States

typedef enum {

DIO_OK = 0,                 // Operation successful

DIO_ERROR_INVALID_PORT,     // Invalid port number

DIO_ERROR_INVALID_PIN,      // Invalid pin number

DIO_ERROR_INVALID_DIR,      // Invalid direction

DIO_ERROR_INVALID_VAL,      // Invalid value

DIO_ERROR_NULL_PTR          // NULL pointer provided

} DIO_ErrorState_t;

Configuration Structures

Pin Configuration Structure:

typedef struct {

u8 port;        // Port (DIO_PORTA - DIO_PORTD)

u8 pin;         // Pin (DIO_PIN0 - DIO_PIN7)

u8 direction;   // Direction (DIO_PIN_INPUT/OUTPUT)

u8 pullup;      // Pull-up (DIO_PULLUP_ENABLE/DISABLE)

} DIO_PinConfig_t;

Port Configuration Structure:

typedef struct {

u8 port;            // Port (DIO_PORTA - DIO_PORTD)

u8 direction;       // Direction (DIO_PORT_INPUT/OUTPUT)

u8 initialValue;    // Initial value for output port

} DIO_PortConfig_t;

Pin-Level Functions

DIO_setPinDirection()

DIO_ErrorState_t DIO_setPinDirection(u8 copy_u8Port, u8 copy_u8Pin, u8 copy_u8Direction);

Sets the direction of a specific pin (input or output).

Parameters:

- copy_u8Port: Port number (DIO_PORTA - DIO_PORTD)

- copy_u8Pin: Pin number (DIO_PIN0 - DIO_PIN7)

- copy_u8Direction: Direction (DIO_PIN_INPUT or DIO_PIN_OUTPUT)

Returns: DIO_OK if successful, error code otherwise

Example:

// Configure PB0 as output

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

// Configure PD2 as input

DIO_setPinDirection(DIO_PORTD, DIO_PIN2, DIO_PIN_INPUT);

DIO_setPinValue()

DIO_ErrorState_t DIO_setPinValue(u8 copy_u8Port, u8 copy_u8Pin, u8 copy_u8Value);

Sets the output value of a pin (high or low).

Parameters:

- copy_u8Port: Port number

- copy_u8Pin: Pin number

- copy_u8Value: Value (DIO_PIN_LOW or DIO_PIN_HIGH)

Returns: DIO_OK if successful, error code otherwise

Example:

// Turn on LED connected to PB0

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_HIGH);

// Turn off LED

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_LOW);

DIO_togglePinValue()

DIO_ErrorState_t DIO_togglePinValue(u8 copy_u8Port, u8 copy_u8Pin);

Toggles the current output value of a pin.

Parameters:

- copy_u8Port: Port number

- copy_u8Pin: Pin number

Returns: DIO_OK if successful, error code otherwise

Example:

// Blink LED on PB0

while(1) {

DIO_togglePinValue(DIO_PORTB, DIO_PIN0);

_delay_ms(500);

}

DIO_readPinValue()

DIO_ErrorState_t DIO_readPinValue(u8 copy_u8Port, u8 copy_u8Pin, u8* copy_pu8Value);

Reads the current value of an input pin.

Parameters:

- copy_u8Port: Port number

- copy_u8Pin: Pin number

- copy_pu8Value: Pointer to store the read value

Returns: DIO_OK if successful, error code otherwise

Example:

u8 buttonState;

DIO_readPinValue(DIO_PORTD, DIO_PIN2, &buttonState);

if(buttonState == DIO_PIN_LOW) {

// Button is pressed

}

DIO_configurePullup()

DIO_ErrorState_t DIO_configurePullup(u8 copy_u8Port, u8 copy_u8Pin, u8 copy_u8Enable);

Enables or disables the internal pull-up resistor for an input pin.

Parameters:

- copy_u8Port: Port number

- copy_u8Pin: Pin number

- copy_u8Enable: DIO_PULLUP_ENABLE or DIO_PULLUP_DISABLE

Returns: DIO_OK if successful, error code otherwise

Example:

// Configure PD2 as input with pull-up enabled

DIO_setPinDirection(DIO_PORTD, DIO_PIN2, DIO_PIN_INPUT);

DIO_configurePullup(DIO_PORTD, DIO_PIN2, DIO_PULLUP_ENABLE);

DIO_initPins()

DIO_ErrorState_t DIO_initPins(const DIO_PinConfig_t* copy_pinConfigs, u8 copy_u8Count);

Initializes multiple pins using a configuration array.

Parameters:

- copy_pinConfigs: Pointer to array of pin configurations

- copy_u8Count: Number of configurations in array

Returns: DIO_OK if successful, error code otherwise

Example:

const DIO_PinConfig_t myPins[] = {

{DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT, DIO_PULLUP_DISABLE},  // LED

{DIO_PORTB, DIO_PIN1, DIO_PIN_OUTPUT, DIO_PULLUP_DISABLE},  // Buzzer

{DIO_PORTD, DIO_PIN2, DIO_PIN_INPUT,  DIO_PULLUP_ENABLE},   // Button

{DIO_PORTD, DIO_PIN3, DIO_PIN_INPUT,  DIO_PULLUP_ENABLE}    // Switch

};

DIO_initPins(myPins, 4);

Port-Level Functions

DIO_setPortDirection()

DIO_ErrorState_t DIO_setPortDirection(u8 copy_u8Port, u8 copy_u8Direction);

Sets the direction of an entire port.

Parameters:

- copy_u8Port: Port number

- copy_u8Direction: DIO_PORT_INPUT or DIO_PORT_OUTPUT

Returns: DIO_OK if successful, error code otherwise

Example:

// Set PORTC as output

DIO_setPortDirection(DIO_PORTC, DIO_PORT_OUTPUT);

// Set PORTD as input

DIO_setPortDirection(DIO_PORTD, DIO_PORT_INPUT);

DIO_setPortValue()

DIO_ErrorState_t DIO_setPortValue(u8 copy_u8Port, u8 copy_u8Value);

Sets the value of an entire output port.

Parameters:

- copy_u8Port: Port number

- copy_u8Value: 8-bit value to write to the port

Returns: DIO_OK if successful, error code otherwise

Example:

// Set all PORTC pins high

DIO_setPortValue(DIO_PORTC, 0xFF);

// Set PORTC to alternating pattern

DIO_setPortValue(DIO_PORTC, 0xAA);  // 10101010

DIO_readPortValue()

DIO_ErrorState_t DIO_readPortValue(u8 copy_u8Port, u8* copy_pu8Value);

Reads the current value of an input port.

Parameters:

- copy_u8Port: Port number

- copy_pu8Value: Pointer to store the read value

Returns: DIO_OK if successful, error code otherwise

Example:

u8 portDValue;

DIO_readPortValue(DIO_PORTD, &portDValue);

DIO_togglePortValue()

DIO_ErrorState_t DIO_togglePortValue(u8 copy_u8Port);

Toggles all bits of an output port.

Parameters:

- copy_u8Port: Port number

Returns: DIO_OK if successful, error code otherwise

Example:

// Toggle all pins on PORTC

DIO_togglePortValue(DIO_PORTC);

DIO_setPortMasked()

DIO_ErrorState_t DIO_setPortMasked(u8 copy_u8Port, u8 copy_u8Mask, u8 copy_u8Value);

Modifies specific bits of a port using a mask.

Parameters:

- copy_u8Port: Port number

- copy_u8Mask: Bit mask (1s indicate bits to modify)

- copy_u8Value: Value to set for masked bits

Returns: DIO_OK if successful, error code otherwise

Example:

// Set upper nibble of PORTC to 0xA0, preserve lower nibble

DIO_setPortMasked(DIO_PORTC, 0xF0, 0xA0);

// Set only pin 3 of PORTB high

DIO_setPortMasked(DIO_PORTB, (1 << DIO_PIN3), 0xFF);

DIO_initPort()

DIO_ErrorState_t DIO_initPort(const DIO_PortConfig_t* copy_config);

Initializes a port with full configuration.

Parameters:

- copy_config: Pointer to port configuration structure

Returns: DIO_OK if successful, error code otherwise

Example:

DIO_PortConfig_t portCConfig = {

.port = DIO_PORTC,

.direction = DIO_PORT_OUTPUT,

.initialValue = 0x00

};

DIO_initPort(&portCConfig);

Utility Functions

DIO_getPortRegisters()

DIO_ErrorState_t DIO_getPortRegisters(u8 copy_u8Port, volatile u8** copy_pDDR,

volatile u8** copy_pPORT, volatile u8** copy_pPIN);

Gets pointers to the DDR, PORT, and PIN registers for a port.

Parameters:

- copy_u8Port: Port number

- copy_pDDR: Pointer to store DDR register address

- copy_pPORT: Pointer to store PORT register address

- copy_pPIN: Pointer to store PIN register address

Returns: DIO_OK if successful, error code otherwise

Example:

volatile u8* ddr;

volatile u8* port;

volatile u8* pin;

if(DIO_getPortRegisters(DIO_PORTB, &ddr, &port, &pin) == DIO_OK) {

// Direct register access

*ddr |= (1 << 0);  // Set PB0 as output

}

DIO_isValidPort()

bool_t DIO_isValidPort(u8 copy_u8Port);

Checks if a port number is valid.

Returns: TRUE if valid, FALSE otherwise

Example:

if(DIO_isValidPort(portNumber)) {

// Safe to use this port

}

DIO_isValidPin()

bool_t DIO_isValidPin(u8 copy_u8Pin);

Checks if a pin number is valid.

Returns: TRUE if valid, FALSE otherwise

Example:

if(DIO_isValidPin(pinNumber)) {

// Safe to use this pin

}

Code Examples

Example 1: LED Blinking

#include "MCAL/DIO_PROGRAM.h"

#include <util/delay.h>

int main(void) {

// Configure LED pin

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

while(1) {

// Turn LED on

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_HIGH);

_delay_ms(500);

// Turn LED off

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_LOW);

_delay_ms(500);

// Or use toggle for cleaner code

// DIO_togglePinValue(DIO_PORTB, DIO_PIN0);

// _delay_ms(500);

}

}

Example 2: Button Reading with Pull-up

#include "MCAL/DIO_PROGRAM.h"

#include <util/delay.h>

int main(void) {

u8 buttonState;

// Configure button pin (input with pull-up)

DIO_setPinDirection(DIO_PORTD, DIO_PIN2, DIO_PIN_INPUT);

DIO_configurePullup(DIO_PORTD, DIO_PIN2, DIO_PULLUP_ENABLE);

// Configure LED pin (output)

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

while(1) {

// Read button state

DIO_readPinValue(DIO_PORTD, DIO_PIN2, &buttonState);

// Button pressed? (active low with pull-up)

if(buttonState == DIO_PIN_LOW) {

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_HIGH);  // LED on

} else {

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_LOW);   // LED off

}

_delay_ms(10);  // Debounce delay

}

}

Example 3: 7-Segment Display Driver

#include "MCAL/DIO_PROGRAM.h"

// 7-segment display patterns (common cathode)

const u8 sevenSegPatterns[] = {

0x3F,  // 0

0x06,  // 1

0x5B,  // 2

0x4F,  // 3

0x66,  // 4

0x6D,  // 5

0x7D,  // 6

0x07,  // 7

0x7F,  // 8

0x6F   // 9

};

int main(void) {

u8 digit = 0;

// Configure PORTC as output for 7-segment data

DIO_setPortDirection(DIO_PORTC, DIO_PORT_OUTPUT);

// Configure control pins

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);  // Digit select 1

DIO_setPinDirection(DIO_PORTB, DIO_PIN1, DIO_PIN_OUTPUT);  // Digit select 2

while(1) {

// Display digit on first display

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_HIGH);   // Enable digit 1

DIO_setPinValue(DIO_PORTB, DIO_PIN1, DIO_PIN_LOW);    // Disable digit 2

DIO_setPortValue(DIO_PORTC, sevenSegPatterns[digit]);

_delay_ms(5);

// Display digit on second display

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_LOW);    // Disable digit 1

DIO_setPinValue(DIO_PORTB, DIO_PIN1, DIO_PIN_HIGH);   // Enable digit 2

DIO_setPortValue(DIO_PORTC, sevenSegPatterns[9-digit]);

_delay_ms(5);

digit++;

if(digit > 9) digit = 0;

}

}

Example 4: Multiple Pin Configuration

#include "MCAL/DIO_PROGRAM.h"

#include <util/delay.h>

int main(void) {

// Configure multiple pins at once

const DIO_PinConfig_t systemPins[] = {

// Outputs

{DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT, DIO_PULLUP_DISABLE},  // LED Red

{DIO_PORTB, DIO_PIN1, DIO_PIN_OUTPUT, DIO_PULLUP_DISABLE},  // LED Green

{DIO_PORTB, DIO_PIN2, DIO_PIN_OUTPUT, DIO_PULLUP_DISABLE},  // LED Blue

{DIO_PORTC, DIO_PIN0, DIO_PIN_OUTPUT, DIO_PULLUP_DISABLE},  // Buzzer

// Inputs with pull-ups

{DIO_PORTD, DIO_PIN2, DIO_PIN_INPUT, DIO_PULLUP_ENABLE},    // Button 1

{DIO_PORTD, DIO_PIN3, DIO_PIN_INPUT, DIO_PULLUP_ENABLE},    // Button 2

{DIO_PORTD, DIO_PIN4, DIO_PIN_INPUT, DIO_PULLUP_ENABLE},    // Switch

};

if(DIO_initPins(systemPins, 7) != DIO_OK) {

// Handle initialization error

while(1);

}

while(1) {

u8 button1, button2;

// Read buttons

DIO_readPinValue(DIO_PORTD, DIO_PIN2, &button1);

DIO_readPinValue(DIO_PORTD, DIO_PIN3, &button2);

// Control LEDs based on buttons

DIO_setPinValue(DIO_PORTB, DIO_PIN0, button1);

DIO_setPinValue(DIO_PORTB, DIO_PIN1, button2);

_delay_ms(50);

}

}

Example 5: Port Masking for LCD-like Interface

#include "MCAL/DIO_PROGRAM.h"

// Simulate 4-bit LCD interface using masked operations

void lcdWriteNibble(u8 data) {

// Assuming data pins are on PORTB pins 4-7

// Preserve lower nibble, update upper nibble with data

DIO_setPortMasked(DIO_PORTB, 0xF0, data << 4);

// Generate enable pulse (PB3)

DIO_setPinValue(DIO_PORTB, DIO_PIN3, DIO_PIN_HIGH);

_delay_us(1);

DIO_setPinValue(DIO_PORTB, DIO_PIN3, DIO_PIN_LOW);

_delay_us(100);

}

int main(void) {

// Configure control pins

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);  // RS

DIO_setPinDirection(DIO_PORTB, DIO_PIN1, DIO_PIN_OUTPUT);  // RW

DIO_setPinDirection(DIO_PORTB, DIO_PIN2, DIO_PIN_OUTPUT);  // E

// Configure data pins (PB4-PB7) as outputs while preserving others

DIO_setPortMasked(DIO_PORTB, 0xF0, 0x00);  // Set data pins low

DIO_setPortDirection(DIO_PORTB, 0xF0);     // Set data pins as output

// Send command 0x28 (4-bit mode, 2 lines)

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_LOW);  // RS = 0 (command)

DIO_setPinValue(DIO_PORTB, DIO_PIN1, DIO_PIN_LOW);  // RW = 0 (write)

lcdWriteNibble(0x02);  // Function set high nibble

lcdWriteNibble(0x08);  // Function set low nibble

while(1);

}

Example 6: Reading DIP Switch Bank

#include "MCAL/DIO_PROGRAM.h"

#include <util/delay.h>

int main(void) {

u8 switchValue;

// Configure PORTD as input for 8 DIP switches

DIO_setPortDirection(DIO_PORTD, DIO_PORT_INPUT);

// Enable pull-ups on all pins

for(u8 i = 0; i < 8; i++) {

DIO_configurePullup(DIO_PORTD, i, DIO_PULLUP_ENABLE);

}

// Configure PORTC as output for LEDs

DIO_setPortDirection(DIO_PORTC, DIO_PORT_OUTPUT);

while(1) {

// Read all switches

DIO_readPortValue(DIO_PORTD, &switchValue);

// Invert because pull-ups make switches active low

switchValue = ~switchValue;

// Display on LEDs

DIO_setPortValue(DIO_PORTC, switchValue);

_delay_ms(10);

}

}

Example 7: Error Handling Demonstration

#include "MCAL/DIO_PROGRAM.h"

#include <util/delay.h>

void handleError(DIO_ErrorState_t error) {

switch(error) {

case DIO_OK:

// No error

break;

case DIO_ERROR_INVALID_PORT:

// Blink LED rapidly for invalid port

for(u8 i = 0; i < 10; i++) {

DIO_togglePinValue(DIO_PORTB, DIO_PIN0);

_delay_ms(50);

}

break;

case DIO_ERROR_INVALID_PIN:

// Blink LED slowly for invalid pin

for(u8 i = 0; i < 5; i++) {

DIO_togglePinValue(DIO_PORTB, DIO_PIN0);

_delay_ms(200);

}

break;

case DIO_ERROR_NULL_PTR:

// Long blink for NULL pointer

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_HIGH);

_delay_ms(1000);

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_LOW);

break;

default:

// Unknown error - fast blinking

while(1) {

DIO_togglePinValue(DIO_PORTB, DIO_PIN0);

_delay_ms(100);

}

}

}

int main(void) {

DIO_ErrorState_t ret;

u8 value;

// Setup error indicator LED

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

// Test with invalid port

ret = DIO_setPinDirection(5, DIO_PIN0, DIO_PIN_OUTPUT);

if(ret != DIO_OK) {

handleError(ret);

}

// Test with NULL pointer

ret = DIO_readPinValue(DIO_PORTA, DIO_PIN0, NULL);

if(ret != DIO_OK) {

handleError(ret);

}

// Valid operation

ret = DIO_setPinDirection(DIO_PORTA, DIO_PIN0, DIO_PIN_OUTPUT);

if(ret == DIO_OK) {

// Configured successfully

DIO_setPinValue(DIO_PORTA, DIO_PIN0, DIO_PIN_HIGH);

}

while(1);

}

Error Handling

Error Codes Reference

| Error Code | Description | When It Occurs |
| --- | --- | --- |
| DIO_OK | Operation successful | No error |
| DIO_ERROR_INVALID_PORT | Invalid port number | Port > DIO_PORTD |
| DIO_ERROR_INVALID_PIN | Invalid pin number | Pin > DIO_PIN7 |
| DIO_ERROR_INVALID_DIR | Invalid direction | Direction not 0 or 1 |
| DIO_ERROR_INVALID_VAL | Invalid value | Value not 0 or 1 |
| DIO_ERROR_NULL_PTR | NULL pointer | Pointer parameter is NULL |

Error Handling Pattern

Always check return values, especially in initialization code:

DIO_ErrorState_t ret;

ret = DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

if(ret != DIO_OK) {

// Handle error appropriately

while(1);  // Halt on critical error

}

Best Practices

1. Always Validate Parameters

// Good - driver handles validation

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

// Bad - assuming valid input

// Direct register access without validation

2. Check Return Values

// Good

if(DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT) != DIO_OK) {

// Handle error

}

// Bad - ignoring errors

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);  // No error check

3. Use Configuration Structures for Multiple Pins

// Good - organized and efficient

const DIO_PinConfig_t config[] = {...};

DIO_initPins(config, 5);

// Bad - repetitive code

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

DIO_setPinDirection(DIO_PORTB, DIO_PIN1, DIO_PIN_OUTPUT);

// ... more repetitive code

4. Configure Pull-ups Appropriately

// For buttons/switches (active low)

DIO_setPinDirection(DIO_PORTD, DIO_PIN2, DIO_PIN_INPUT);

DIO_configurePullup(DIO_PORTD, DIO_PIN2, DIO_PULLUP_ENABLE);

// For floating inputs, always enable pull-up or connect externally

5. Use Atomic Operations When Needed

The driver automatically handles atomic operations, so you don't need to worry about interrupts corrupting register writes.

6. Initialize Before Use

// Always set direction before reading/writing

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);  // First

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_HIGH);       // Then

Troubleshooting

Common Issues and Solutions

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| Pin doesn't change state | Direction not set to output | Call DIO_setPinDirection() first |
| Button always reads HIGH | Pull-up not enabled | Enable pull-up with DIO_configurePullup() |
| Intermittent behavior | No pull-up on input | Enable internal or external pull-up |
| Wrong pin toggling | Pin number incorrect | Verify pin numbering (0-7) |
| Multiple pins affected | Using wrong port | Check port selection |
| Function returns error | Invalid parameters | Validate port/pin values |
| LED too dim | Pin current limit | Add external transistor for high current |

Debugging Tips

- Verify Pin Configuration

// Read back pin state to verify

u8 pinValue;

DIO_readPinValue(DIO_PORTB, DIO_PIN0, &pinValue);

// pinValue should reflect output state

- Check Return Values

DIO_ErrorState_t ret = DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

if(ret != DIO_OK) {

// Breakpoint or LED indicator here

}

- Use Validation Functions

if(!DIO_isValidPort(portNum)) {

// Port number is invalid

}

Migration Guide

From Old Driver to New Driver

| Old Function | New Function | Notes |
| --- | --- | --- |
| DIO_setPinDirection() | DIO_setPinDirection() | Added return value |
| DIO_setPinVal() | DIO_setPinValue() | Renamed, added return |
| DIO_togglePinVal() | DIO_togglePinValue() | Renamed, added return |
| DIO_readPinVal() | DIO_readPinValue() | Renamed, added return |
| DIO_setInputInternalPullup() | DIO_configurePullup() | More flexible |
| DIO_setPortDir() | DIO_setPortDirection() | Renamed, added return |
| DIO_setPortVal() | DIO_setPortValue() | Renamed, added return |
| - | DIO_setPortMasked() | New feature |
| - | DIO_initPins() | New feature |
| - | DIO_initPort() | New feature |
| - | DIO_getPortRegisters() | New feature |

Example Migration

// Old code

DIO_setPinDirection(DIO_PORTB, 0, 1);  // Hard to read

DIO_setPinVal(DIO_PORTB, 0, 1);

// New code

DIO_setPinDirection(DIO_PORTB, DIO_PIN0, DIO_PIN_OUTPUT);

DIO_setPinValue(DIO_PORTB, DIO_PIN0, DIO_PIN_HIGH);

Performance Characteristics

- Pin operations: ~2-3 CPU cycles (register access only)

- Port operations: ~1-2 CPU cycles

- Validation overhead: Minimal (few comparisons)

- Atomic operations: Disables interrupts for ~5 cycles

Memory Footprint

- Code size: ~500-800 bytes (depending on usage)

- RAM usage: 0 bytes (no global variables)

- ROM usage: Minimal constant data

Conclusion

This DIO driver provides a robust, professional foundation for all ATmega32A projects. Its comprehensive error handling, atomic operations, and flexible API make it suitable for both simple LED blinking and complex peripheral interfacing.

Document Version: 2.0
Last Updated: February 2026
Author: Basel Mohamed Mostafa Sayed
Compatible With: ATmega32A, AVR-GCC

# Chapter 7: Conclusion

## 7.1 Conclusion

The project successfully met all design requirements. By developing custom, layered C drivers rather than relying on pre-built Arduino libraries, the project demonstrates a deep understanding of AVR register manipulation. The non-blocking FSM allows for a robust, crash-free user experience, proving that a single 8-bit microcontroller can simultaneously handle wireless communications, LCD updates, mechanical sweeping, and spatial mapping.